1. Thrust Formula:

T=12ρAv2CT

Where:

- ρ= Air density (kg/m³)

- A= Effective propeller area (m²), typically related to the propeller diameter

- v= Airspeed or velocity through the propeller (m/s)

- CT= Thrust coefficient, which depends on propeller design, rotation speed, and angle of attack.

2. Thrust Coefficient CTCalculation:

The thrust coefficient CTcan be calculated using an empirical equation based on the propeller's rotation speed, angle of attack, and flow characteristics:

CT=k1vvtip+k2vvtip2

Where:

- vtip= Tip speed of the propeller (m/s), typically calculated as propeller rotational speed multiplied by the radius

- k1,k2= Constants, usually derived from experimental data or theoretical models.

3. Aerodynamic Forces:

Thrust is also affected by the aerodynamic forces (Lift, Drag, Side Force, and Moments) acting on the aircraft. The aerodynamic forces can be calculated using:

- Lift L:

L=12ρv2SCL

Where:

- S= Wing area (m²)

- CL= Lift coefficient, depending on the angle of attack and flight conditions.

- Drag D:

D=12ρv2SCD

Where:

- CD= Drag coefficient, which depends on flight speed, angle of attack, etc.

- Side Force (Side Slip) and Moments are similarly influenced by control surfaces like the rudder, ailerons, and elevator.

4. PID Control for Aircraft Attitude:

PID controllers are used to stabilize the aircraft's roll, pitch, and yaw by adjusting control surfaces based on feedback.

The basic formula for PID control is:

u(t)=KPe(t)+KI0te(τ)dτ+KDddte(t)

Where:

- e(t)= Error (difference between desired and actual attitude, such as roll or pitch angle)

- KP, KI, KD= Proportional, Integral, and Derivative gains, which need to be adjusted for optimal performance.

PID Tuning Process (Ziegler-Nichols Method):

The Ziegler-Nichols method is often used for initial PID tuning:

- Increase KPuntil the system starts to oscillate.

- Measure the oscillation period and use it to calculate the other gains.

- Fine-tune KP, KI, and KDthrough simulations and real tests to achieve the desired stability and response.